ADAPTIVE TRAFFIC SIGNAL SYSTEM

Using Vision + Sensor Fusion + AI Speed Limit Control

ESP32-CAM  ·  ESP32 Dev Board  ·  Python Laptop  ·  Vercel Dashboard

Complete Build Guide — Connections · Phases · Claude Code Prompts · Test File

# SECTION 1 — SYSTEM OVERVIEW

This project is a distributed edge-AI traffic management system. It uses two ESP32 microcontrollers, a laptop running Python and computer vision, a cloud database (Supabase), and a police-facing web dashboard deployed on Vercel.

## 1.1 — The Three-Layer Architecture

- Layer 1 — Edge Hardware: ESP32-CAM captures images and reads IR sensors. ESP32 Dev Board drives the physical traffic lights (LEDs) and OLED speed-limit display.

- Layer 2 — Local Processing: Laptop runs YOLOv8 car detection, sensor fusion, decision engine, Flask API, and pushes data to Supabase.

- Layer 3 — Cloud + Frontend: Supabase stores all traffic events in real time. Vercel hosts a Next.js dashboard for traffic police to monitor live feeds, view history, and override signals.

## 1.2 — Data Flow (end to end)

ESP32-CAM captures JPEG frame + reads IR sensors → sends to Laptop over WiFi → Laptop runs YOLO + fusion + decision → computes signal (RED/YELLOW/GREEN) + speed limit (20/30/40/60/80 km/h) → POSTs command JSON to ESP32 Dev Board → Dev Board sets LEDs + OLED → Laptop also pushes state to Supabase → Vercel dashboard reads Supabase in real time via WebSocket → Police officer sees live feed + can override from browser.

## 1.3 — Command JSON (the spine of the system)

Every decision made by the laptop is encoded in this single JSON object:

{ "signal": "GREEN", "duration": 20, "speed_limit": 40 }

This object flows from Laptop → ESP32 Dev Board via HTTP POST to /cmd. The Dev Board parses it and acts on it immediately.

# SECTION 2 — HARDWARE COMPONENTS

## 2.1 — Full Component List

| Component | Quantity | Purpose |
| --- | --- | --- |
| ESP32-CAM (AI-Thinker) | 1 | Camera capture + IR sensor reading + WiFi |
| ESP32 Dev Board (38-pin) | 1 | LED control + OLED display + WiFi |
| OV2640 Camera Module | 1 | Built into ESP32-CAM (FPC ribbon) |
| SSD1306 OLED 128x64 I2C | 1 | Displays dynamic speed limit |
| IR Obstacle Sensor (FC-51) | 3 | Detects vehicle presence at 25/50/75% queue |
| Red LED 5mm | 1 | Traffic signal — STOP |
| Yellow LED 5mm | 1 | Traffic signal — CAUTION |
| Green LED 5mm | 1 | Traffic signal — GO |
| 220 Ohm resistor | 3 | Current limiting for each LED |
| FTDI232 / CP2102 USB-Serial | 1 | Programming the ESP32-CAM |
| Breadboard (full size) | 1 | Prototyping connections |
| Jumper wires (M-M, M-F) | 30+ | All connections |
| 5V / 2A USB power supply | 1 | Powers ESP32-CAM (camera needs 2A) |

# SECTION 3 — ALL PHYSICAL CONNECTIONS

## 3.1 — ESP32-CAM Pin Constraints (CRITICAL — READ THIS FIRST)

The ESP32-CAM AI-Thinker has most GPIOs occupied by the camera and SD card. Only 3 GPIOs are freely usable after camera initialisation:

| GPIO | Camera/System Use | Status | Our Use |
| --- | --- | --- | --- |
| GPIO 0 | XCLK (camera clock) | LOCKED | — (GND during flash only) |
| GPIO 1 | UART TX | SERIAL DEBUG | Debug output only |
| GPIO 2 | Free | FREE | IR Sensor 1 — 25% (near) |
| GPIO 3 | UART RX | SERIAL DEBUG | Debug output only |
| GPIO 4 | Built-in flash LED | USABLE (flash off) | IR Sensor 2 — 50% (mid) |
| GPIO 12 | Camera data / SD | LOCKED | — |
| GPIO 13 | Camera data / SD | LOCKED | — |
| GPIO 14 | SD clock | LOCKED | — |
| GPIO 15 | SD CMD | LOCKED | — |
| GPIO 16 | Free | FREE | IR Sensor 3 — 75% (far) |
| GPIO 33 | Onboard LED | FREE | Status blink (optional) |

WARNING: Because only 3 GPIOs are free on the ESP32-CAM, ALL OUTPUT DEVICES (LEDs, OLED) connect to the ESP32 Dev Board — NOT the CAM.

## 3.2 — ESP32-CAM Wiring (Sensors + Power)

| From | To (ESP32-CAM Pin) | Wire Color (suggested) | Notes |
| --- | --- | --- | --- |
| IR Sensor 1 OUT | GPIO 2 | Yellow | 25% — nearest to junction |
| IR Sensor 2 OUT | GPIO 4 | Orange | 50% — mid road |
| IR Sensor 3 OUT | GPIO 16 | Blue | 75% — far from junction |
| All IR Sensors VCC | 3.3V pin | Red | All 3 share the 3.3V rail |
| All IR Sensors GND | GND pin | Black | All 3 share GND |
| 5V Supply + | 5V pin | Red | Minimum 2A — camera is power hungry |
| 5V Supply GND | GND pin | Black | Common ground |

## 3.3 — ESP32-CAM Programming Connections (FTDI232)

These connections are ONLY needed during flashing. Remove the GPIO0-GND bridge after flashing.

| FTDI232 Pin | ESP32-CAM Pin | Notes |
| --- | --- | --- |
| TX | GPIO 3 (U0RXD) | Cross-connected: FTDI TX → CAM RX |
| RX | GPIO 1 (U0TXD) | Cross-connected: FTDI RX → CAM TX |
| GND | GND | Common ground |
| 3.3V or 5V | Do NOT power from FTDI | Use dedicated 5V/2A supply |
| GPIO 0 | GND (bridge) | ONLY during flashing — remove after |

WARNING: Do NOT power the ESP32-CAM from the FTDI 3.3V pin. The camera draws up to 800mA and will brownout the FTDI. Always use a separate 5V/2A power supply.

## 3.4 — ESP32 Dev Board Wiring (LEDs + OLED)

The ESP32 Dev Board has abundant free GPIOs. All output devices connect here.

| Component | Component Pin | ESP32 Dev GPIO | Resistor? | Notes |
| --- | --- | --- | --- | --- |
| RED LED | Anode (+) | GPIO 25 | 220 Ohm in series | Cathode (-) to GND |
| YELLOW LED | Anode (+) | GPIO 26 | 220 Ohm in series | Cathode (-) to GND |
| GREEN LED | Anode (+) | GPIO 27 | 220 Ohm in series | Cathode (-) to GND |
| SSD1306 OLED | SDA | GPIO 21 | None | I2C data line |
| SSD1306 OLED | SCL | GPIO 22 | None | I2C clock line |
| SSD1306 OLED | VCC | 3.3V pin | None | OLED runs on 3.3V |
| SSD1306 OLED | GND | GND pin | None | Common ground |

NOTE: LED wiring: Long leg = Anode (+) connects via 220 Ohm resistor to GPIO. Short leg = Cathode (-) connects directly to GND.

NOTE: OLED I2C address is 0x3C by default on most SSD1306 modules. If the display does not initialise, try 0x3D.

## 3.5 — IR Sensor Calibration

Each FC-51 IR sensor has a small blue potentiometer on it. Turn it to set detection distance:

- IR Sensor 1 (GPIO 2): Set to detect ~1-2 car lengths away. This is the NEAR sensor — represents 25% queue fill.

- IR Sensor 2 (GPIO 4): Set to detect ~3-4 car lengths. This is the MID sensor — represents 50% queue fill.

- IR Sensor 3 (GPIO 16): Set to detect ~5-6 car lengths. This is the FAR sensor — represents 75% queue fill.

When a vehicle is within range, the sensor OUTPUT pin goes LOW (0). When clear, OUTPUT goes HIGH (1). The firmware reads this as: LOW = vehicle detected.

# SECTION 4 — HOW EVERYTHING WORKS (DEEP DIVE)

## 4.1 — Phase 1: ESP32-CAM Firmware

What runs on the ESP32-CAM after flashing:

- On boot: ESP32-CAM connects to your WiFi network using hardcoded SSID/password in config.h.

- Starts an HTTP server on port 80 with two endpoints.

- /stream endpoint: When the laptop hits this URL, the CAM captures a JPEG frame from the OV2640 and returns it as image/jpeg. The laptop polls this every 2 seconds.

- /sensors endpoint: Returns a JSON object with the current state of all 3 IR sensors. The laptop polls this every 2 seconds in parallel.

- IR sensors are read every 200ms internally and their state is stored in memory — the /sensors endpoint just reads the last known state.

GET /sensors → {"s25": 1, "s50": 0, "s75": 0}

s25=1 means IR sensor 1 (25%) has detected a vehicle. s50=0 means IR sensor 2 (50%) is clear.

NOTE: The ESP32-CAM does NOT receive commands and does NOT control any outputs. It only serves data. This is intentional — the CAM has no free GPIO for outputs.

## 4.2 — Phase 2: ESP32 Dev Board Firmware

What runs on the ESP32 Dev Board permanently:

- On boot: connects to same WiFi network. Gets an IP address from the router.

- Starts HTTP server on port 80 with /cmd endpoint accepting POST requests.

- When laptop POSTs { "signal": "GREEN", "duration": 20, "speed_limit": 40 }:

  - Sets GPIO 25 (RED) LOW, GPIO 26 (YELLOW) LOW, GPIO 27 (GREEN) HIGH — traffic light shows GREEN.

  - Writes "40 km/h" in large text to the SSD1306 OLED display via I2C.

  - Holds this state until the next command arrives.

- Responds HTTP 200 OK to the laptop.

NOTE: The Dev Board is a pure output device. It has no camera, no sensors, no decision logic. It just obeys commands from the laptop.

## 4.3 — Phase 3: Vision Module (Laptop)

The laptop runs a Python function that:

- Makes an HTTP GET request to http://[ESP32-CAM-IP]/stream

- Receives a JPEG image as bytes

- Passes the image to YOLOv8-nano model (pre-trained on COCO dataset which includes cars, trucks, buses, motorcycles)

- Counts all detected vehicles (class IDs: 2=car, 5=bus, 7=truck, 3=motorcycle)

- Returns car_count as an integer

YOLOv8-nano is used because it runs fast on CPU (~150ms per frame) and is accurate enough for road detection.

## 4.4 — Phase 4: Sensor Fusion + Decision Engine (Laptop)

The fusion function combines camera vision with physical IR sensor data:

queue_level = (s25 * 1) + (s50 * 2) + (s75 * 3)   # max = 6

traffic_score = (car_count * 0.6) + (queue_level * 0.4)

The decision engine maps traffic_score to outputs:

| traffic_score | Signal | Duration | Speed Limit | Meaning |
| --- | --- | --- | --- | --- |
| < 3 | GREEN | 30s | 80 km/h | Road is clear |
| 3 to 7 | GREEN | 25s | 60 km/h | Light traffic |
| 7 to 12 | YELLOW | 20s | 40 km/h | Moderate — slow down |
| 12 to 17 | RED | 20s | 30 km/h | Heavy congestion |
| >= 17 | RED | 30s | 20 km/h | Gridlock — SLOW |

This fusion approach is powerful because it cross-validates: a camera glitch does not trigger wrong decisions if the IR sensors disagree, and vice versa.

## 4.5 — Phase 5: Main Loop + Flask + Supabase + Cloudflare

The main.py orchestrates everything in a continuous loop:

- Step 1: Fetch JPEG from ESP32-CAM /stream

- Step 2: Run YOLO on JPEG → get car_count

- Step 3: Fetch JSON from ESP32-CAM /sensors → get s25, s50, s75

- Step 4: Compute queue_level and traffic_score

- Step 5: Compute signal + duration + speed_limit from score

- Step 6: POST command JSON to ESP32 Dev Board /cmd

- Step 7: Write traffic event row to Supabase (timestamp, car_count, queue_level, traffic_score, signal, speed_limit)

- Step 8: Update heartbeat in Supabase (for online/offline detection)

- Step 9: Wait 2 seconds. Repeat.

Flask API runs in a separate thread and handles manual override POSTs from the Vercel dashboard. An override sets a flag that skips Steps 4-5 for 60 seconds, using the override values instead.

Cloudflare Tunnel is started as a subprocess: it gives the ESP32-CAM stream a public HTTPS URL without needing port forwarding. The URL is written to Supabase so the Vercel frontend can discover it automatically.

## 4.6 — Phase 6: Streamlit Dashboard (Local)

A local Streamlit app running at http://localhost:8501 that shows:

- Live camera feed pulled from ESP32-CAM /stream

- Three coloured dots showing IR sensor states (green=clear, red=vehicle detected)

- Traffic score as a numeric gauge

- Current signal state (large coloured badge)

- Current speed limit

- Auto-refreshes every 2 seconds

This is your local debugging and demo screen. Judges can see everything at once without needing internet.

## 4.7 — Phase 7: Vercel Next.js Police Dashboard

A Next.js app deployed to Vercel at a public URL. Features:

- Live camera feed: Fetches Cloudflare tunnel URL from Supabase config table, embeds MJPEG stream as <img> tag. Police see live road footage.

- Realtime state panel: Supabase Realtime WebSocket pushes every new traffic_events row instantly. Dashboard shows current signal, speed limit, car count, queue level, traffic score — all live without page reload.

- Manual signal override: Three buttons (RED / YELLOW / GREEN). Clicking POSTs to Flask API on the laptop. Override lasts 60 seconds then AI resumes.

- Manual speed limit override: Dropdown (20 / 30 / 40 / 60 / 80). Same override logic.

- Traffic history chart: Line chart of traffic_score over the last 30 minutes using data from Supabase.

- History log table: Scrollable table of all recent decisions with timestamps.

- Congestion alert banner: Red flashing banner when traffic_score >= 17.

- System health indicator: Green dot = online (heartbeat fresh). Red dot = offline (heartbeat stale > 15s).

# SECTION 5 — PHASE-BY-PHASE BUILD GUIDE

## PHASE 1 — ESP32-CAM Firmware

### What to expect

After this phase: ESP32-CAM boots, connects to WiFi, serves camera frames at /stream and IR sensor states at /sensors. You can verify both endpoints from a browser on your laptop.

### Hardware setup for Phase 1

- Connect FTDI232: TX→GPIO3, RX→GPIO1, GND→GND. Do NOT connect 3.3V from FTDI.

- Connect 5V/2A supply to ESP32-CAM 5V and GND pins.

- Bridge GPIO0 to GND (this puts ESP32-CAM into flash mode).

- Connect IR sensors: Sensor1 OUT→GPIO2, Sensor2 OUT→GPIO4, Sensor3 OUT→GPIO16. All VCC→3.3V, all GND→GND.

- Do NOT connect any LEDs or OLED to the CAM — no free pins.

### Arduino IDE setup

- Install board: ESP32 by Espressif (v2.x) in Board Manager.

- Select board: AI Thinker ESP32-CAM.

- Select port: your FTDI232 COM port.

- Upload speed: 115200.

### Flashing procedure

- GPIO0 bridged to GND → power on (or press RESET) → upload from Arduino IDE.

- Wait for "Done uploading" message.

- Remove GPIO0-GND bridge.

- Press RESET button on ESP32-CAM.

- Open Serial Monitor at 115200 baud. You should see WiFi connecting, then "Camera Ready! IP: 192.168.x.x"

WARNING: After flashing, the ESP32-CAM is considered SEALED. You should not need to reflash. All future changes happen on the laptop or Dev Board.

### Testing Phase 1

- Open browser on laptop on same WiFi. Go to http://[IP-from-serial]/stream — you should see a camera frame (static JPEG, not video).

- Go to http://[IP-from-serial]/sensors — you should see {"s25":0,"s50":0,"s75":0} or similar.

- Wave hand in front of each IR sensor — values should flip to 1 when detected.

- Note down the IP address — you will put it in laptop config.py.

TIP: Take a photo of the Serial Monitor showing the IP address. You will need it for every subsequent phase.

CLAUDE CODE PROMPT — PHASE 1A — ESP32-CAM Core Setup

I am building an Adaptive Traffic Signal System using an ESP32-CAM AI-Thinker module.
HARDWARE:
- ESP32-CAM AI-Thinker with OV2640 camera (built-in FPC ribbon)
- 3x IR obstacle sensors (FC-51): OUT pins connected to GPIO 2, GPIO 4, GPIO 16
- All IR sensor VCC connected to ESP32-CAM 3.3V pin
- All IR sensor GND connected to ESP32-CAM GND
TASK: Create the complete Arduino sketch for the ESP32-CAM with the following:
1. config.h file:
   - WiFi SSID and password constants
   - IR sensor GPIO pin definitions: IR_NEAR=2, IR_MID=4, IR_FAR=16
   - HTTP server port: 80
2. camera_config.h file:
   - Full OV2640 camera pin configuration for AI-Thinker board
   - Resolution: FRAMESIZE_VGA (640x480) for good balance of speed and quality
   - JPEG quality: 12 (good quality)
   - Frame buffer count: 1
3. main.ino file:
   - WiFi connection with retry logic (retry every 5s, print dots to serial)
   - Camera initialisation with error handling
   - HTTP server with two endpoints:
     a. GET /stream — captures one JPEG frame, returns as image/jpeg content-type
     b. GET /sensors — reads GPIO 2, 4, 16 (INPUT_PULLUP, LOW=detected) and returns JSON:
        {"s25": 0_or_1, "s50": 0_or_1, "s75": 0_or_1}
   - IR sensor polling loop every 200ms storing state in global variables
   - Serial output at 115200: print IP address on successful WiFi connect
   - Onboard LED (GPIO 33) blinks every second to show system is alive
Requirements:
- Use WebServer.h library for HTTP server
- Use esp_camera.h for camera
- Must handle camera init failure gracefully (print error, halt)
- No SD card code — SD is disabled
- GPIO 0 is NOT used after boot
- Comments explaining each section clearly
Output all three files with full code.

CLAUDE CODE PROMPT — PHASE 1B — ESP32-CAM Testing Verification

The ESP32-CAM firmware from Phase 1A is flashed and running.
Create a Python test script (test_esp32cam.py) that:
1. Takes ESP32-CAM IP as a command line argument or prompts for it
2. Tests /stream endpoint:
   - Makes GET request with 5s timeout
   - Checks response is 200 and content-type is image/jpeg
   - Saves the JPEG as test_frame.jpg
   - Prints image dimensions using PIL
3. Tests /sensors endpoint:
   - Makes GET request
   - Validates JSON has keys s25, s50, s75
   - All values are 0 or 1
   - Prints current sensor states in a readable format
4. Runs both tests 3 times with 2s between runs
5. Prints PASS/FAIL for each test with timing
Requirements: requests, PIL libraries only.
Print clear success/failure messages for each test.

## PHASE 2 — ESP32 Dev Board Firmware

### What to expect

After this phase: ESP32 Dev Board connects to WiFi, exposes /cmd endpoint, and correctly drives LEDs and OLED when you send test commands from a browser or Postman. This phase is fully testable without the CAM or laptop Python code running.

### Hardware setup for Phase 2

- RED LED: Anode → 220 Ohm resistor → GPIO 25. Cathode → GND.

- YELLOW LED: Anode → 220 Ohm resistor → GPIO 26. Cathode → GND.

- GREEN LED: Anode → 220 Ohm resistor → GPIO 27. Cathode → GND.

- SSD1306 OLED: VCC → 3.3V, GND → GND, SDA → GPIO 21, SCL → GPIO 22.

- Connect ESP32 Dev Board via USB to laptop for programming (normal USB — no FTDI needed).

### Arduino IDE setup for Dev Board

- Select board: ESP32 Dev Module (or your specific ESP32 board name).

- Select port: the USB COM port for the Dev Board.

- Upload speed: 921600.

- Install library: Adafruit SSD1306 (via Library Manager). Also install Adafruit GFX Library dependency.

- Install library: ArduinoJson by Benoit Blanchon.

### Testing Phase 2

- After upload, open Serial Monitor at 115200. Should see WiFi connected + IP address.

- Open Postman or use curl to POST to http://[DEV-BOARD-IP]/cmd:

curl -X POST http://[IP]/cmd -H "Content-Type: application/json" -d "{\"signal\":\"RED\",\"duration\":20,\"speed_limit\":30}"

- RED LED should light up. OLED should show "30" and "km/h".

- Test GREEN: same command with "signal":"GREEN", "speed_limit":60. Green LED on, OLED shows 60.

- Test YELLOW similarly.

TIP: Test all three signals before moving to Phase 3. If OLED is blank, check I2C address (try 0x3C then 0x3D in code).

CLAUDE CODE PROMPT — PHASE 2A — ESP32 Dev Board Core

I am building an Adaptive Traffic Signal System. The ESP32 Dev Board is the OUTPUT CONTROLLER.
HARDWARE connected to ESP32 Dev Board:
- RED LED: GPIO 25 (with 220 Ohm series resistor)
- YELLOW LED: GPIO 26 (with 220 Ohm series resistor)
- GREEN LED: GPIO 27 (with 220 Ohm series resistor)
- SSD1306 OLED 128x64 I2C: SDA=GPIO21, SCL=GPIO22, VCC=3.3V
- ESP32 connects to laptop via WiFi (same local network)
TASK: Create complete Arduino firmware for the ESP32 Dev Board:
1. config.h:
   - WiFi credentials
   - LED pin definitions: RED_PIN=25, YEL_PIN=26, GRN_PIN=27
   - I2C pins: SDA_PIN=21, SCL_PIN=22
   - OLED I2C address: 0x3C
   - HTTP server port: 80
2. led_control.h:
   - setSignal(String signal) function: turns on the correct LED, turns off others
   - Signals: "RED", "YELLOW", "GREEN"
   - Safe: any unknown signal string turns ALL LEDs off
3. oled_display.h:
   - initOLED() function
   - showSpeedLimit(int speed) function:
     * Clears display
     * Shows "SPEED LIMIT" in small text at top
     * Shows the speed number in VERY LARGE text (font size 4) centered
     * Shows "km/h" in medium text below
     * If speed is 20, display also shows "SLOW" warning
   - showStartup() function: shows "TRAFFIC AI" and "SYSTEM READY"
   - showOffline() function: shows "OFFLINE" when no command received in 30s
4. main.ino:
   - WiFi connect with retry
   - OLED init and show startup screen
   - HTTP server with POST /cmd endpoint:
     * Parses JSON body for "signal" (string), "duration" (int), "speed_limit" (int)
     * Calls setSignal() and showSpeedLimit()
     * Updates lastCommandTime timestamp
     * Returns HTTP 200 with JSON {"status":"ok"}
     * Returns HTTP 400 with {"error":"invalid json"} on parse failure
   - Background task: if no command received in 30 seconds, show OFFLINE on OLED and turn all LEDs off
   - Serial output: print IP on connect, print each received command
Libraries: WebServer.h, ArduinoJson.h, Adafruit_SSD1306.h, Adafruit_GFX.h, Wire.h
Output all files with full code.

CLAUDE CODE PROMPT — PHASE 2B — Dev Board Test Script

Create a Python test script (test_devboard.py) to fully verify the ESP32 Dev Board firmware.
The script should:
1. Accept Dev Board IP as argument or prompt
2. Test each signal state:
   - POST {"signal":"RED","duration":20,"speed_limit":20} → check HTTP 200
   - Wait 2s (user visually confirms RED LED on, OLED shows 20)
   - POST {"signal":"YELLOW","duration":15,"speed_limit":30}
   - Wait 2s
   - POST {"signal":"GREEN","duration":25,"speed_limit":60}
   - Wait 2s
3. Test speed limits (send GREEN with each speed: 20,30,40,60,80)
4. Test invalid JSON handling (send malformed JSON, expect 400)
5. Test offline detection: wait 35 seconds without sending command, then ask user if OLED shows OFFLINE
6. Print a final test report: PASS/FAIL for each test case
Use only requests library.

## PHASE 3 — Laptop Vision Module

### What to expect

After this phase: A Python function that accepts an IP address and returns car_count. You test it first with a static image, then live from the ESP32-CAM stream. Completely standalone — no Dev Board needed yet.

### Environment setup

- Create virtual environment: python -m venv venv

- Activate: source venv/bin/activate (Mac/Linux) or venv\Scripts\activate (Windows)

- Install: pip install ultralytics requests Pillow opencv-python

- First run of YOLOv8 will download the yolov8n.pt model file (~6MB) automatically.

### Testing Phase 3

- Run test_vision.py with a road image — you can download any traffic photo from Google for testing.

- Expected output: car_count=3 (or however many vehicles are in the image).

- Then test with live ESP32-CAM: python test_vision.py --live --ip [CAM-IP]

- You should see car_count printed every 2 seconds.

TIP: Point the camera at a road image on your phone/monitor for testing if real road is not available.

CLAUDE CODE PROMPT — PHASE 3A — Vision Module

I am building the laptop-side processing for an Adaptive Traffic Signal System.
TASK: Create the vision module that detects vehicles in images from the ESP32-CAM.
FILES TO CREATE:
1. vision.py:
   - load_model() function: loads YOLOv8n model (yolov8n.pt), caches globally after first load
   - detect_vehicles(image_bytes: bytes) -> int function:
     * Accepts raw JPEG bytes from ESP32-CAM
     * Converts to numpy array via PIL + numpy
     * Runs YOLOv8 inference
     * Counts detections with class IDs: 2 (car), 3 (motorcycle), 5 (bus), 7 (truck)
     * Confidence threshold: 0.4
     * Returns total vehicle count as integer
   - fetch_frame(cam_ip: str) -> bytes function:
     * GET request to http://[cam_ip]/stream
     * Timeout: 5 seconds
     * Returns raw JPEG bytes
     * Raises ConnectionError if request fails
2. test_vision.py:
   - Argument parser: --ip (ESP32-CAM IP), --image (path to local test image), --live (flag for continuous mode)
   - If --image: load file, run detect_vehicles, print count, save annotated image
   - If --live --ip: loop every 2s, fetch from ESP32-CAM, detect, print "Vehicle count: N" with timestamp
   - If neither: use a built-in test with a placeholder (print instructions)
   - Annotated image: draw bounding boxes with class name and confidence on detected vehicles
   - Save annotated output as detected_output.jpg
Use: ultralytics, PIL, requests, numpy, cv2 libraries.

## PHASE 4 — Fusion + Decision Engine

### What to expect

After this phase: A Python module that takes car_count and sensor states, returns signal + duration + speed_limit. Fully testable with mock data — no hardware needed.

### Testing Phase 4

- Run test_fusion.py — it runs 10 predefined test cases covering all score ranges.

- All 10 should PASS. If any fail, the thresholds or formulas have a bug.

- The test cases cover edge cases: all sensors off + 0 cars (score=0), all sensors on + many cars (score>>17), etc.

CLAUDE CODE PROMPT — PHASE 4A — Fusion and Decision

Create the fusion and decision engine for the Adaptive Traffic Signal System.
FILES TO CREATE:
1. fusion.py:
   - compute_queue_level(s25: int, s50: int, s75: int) -> int:
     * s25, s50, s75 are 0 or 1
     * Returns: (s25 * 1) + (s50 * 2) + (s75 * 3)
     * Max value = 6 (all sensors triggered)
   - compute_traffic_score(car_count: int, queue_level: int) -> float:
     * Returns: (car_count * 0.6) + (queue_level * 0.4)
     * Cap car_count at 20 before computing (prevents extreme values)
   - fetch_sensor_state(cam_ip: str) -> dict:
     * GET http://[cam_ip]/sensors with 5s timeout
     * Returns dict: {"s25": int, "s50": int, "s75": int}
     * On error: returns {"s25": 0, "s50": 0, "s75": 0} and logs warning
2. decision.py:
   - compute_decision(traffic_score: float, override: dict = None) -> dict:
     * If override is not None and not expired: return override values
     * Score thresholds:
       - < 3: signal=GREEN, duration=30, speed_limit=80
       - 3-7: signal=GREEN, duration=25, speed_limit=60
       - 7-12: signal=YELLOW, duration=20, speed_limit=40
       - 12-17: signal=RED, duration=20, speed_limit=30
       - >= 17: signal=RED, duration=30, speed_limit=20
     * Returns: {"signal": str, "duration": int, "speed_limit": int, "score": float, "is_override": bool}
   - Override logic: override dict has {"signal","speed_limit","expires_at"} where expires_at is datetime
3. test_fusion.py:
   - 10 test cases covering all score ranges and edge cases
   - Test cases: (car_count, s25, s50, s75, expected_signal, expected_speed)
   - Include: (0,0,0,0 → GREEN,80), (5,1,0,0 → GREEN,60), (10,1,1,0 → YELLOW,40),
              (15,1,1,1 → RED,30), (20,1,1,1 → RED,20), override active test,
              override expired test, max values test, zero sensors + high cars test
   - Print PASS/FAIL for each with actual vs expected values
   - Print final score: X/10 passed

## PHASE 5 — Main Loop + Flask + Supabase + Cloudflare

### What to expect

After this phase: The full system runs. Laptop orchestrates everything in a loop. Physical LEDs change. OLED updates. All events are logged to Supabase. Cloudflare tunnel gives the camera a public URL.

### Supabase setup (do this before coding)

- Go to supabase.com, create a free project.

- In Table Editor, create table: traffic_events with columns:

  - id (uuid, primary key, default: gen_random_uuid())

  - created_at (timestamptz, default: now())

  - car_count (int4)

  - queue_level (int4)

  - traffic_score (float8)

  - signal (text)

  - speed_limit (int4)

  - is_override (bool, default: false)

- Create second table: system_state with columns: key (text, primary key), value (text), updated_at (timestamptz).

- Go to Settings → API. Copy your Project URL and anon/public key. Put these in .env file.

- Enable Realtime on traffic_events table: Table Editor → traffic_events → Realtime toggle ON.

### Cloudflare tunnel setup

- Install cloudflared: brew install cloudflared (Mac) or download from cloudflare.com/products/tunnel.

- Run: cloudflared tunnel --url http://[ESP32-CAM-IP]:80

- Copy the generated HTTPS URL (e.g. https://abc-def.trycloudflare.com). This proxies to your ESP32-CAM.

- This URL goes into your Supabase system_state table as key=tunnel_url.

NOTE: The free Cloudflare tunnel URL changes every restart. For the hackathon demo, keep the tunnel running. For production, use a named tunnel.

### Testing Phase 5

- Start main.py. Watch terminal — should print car_count and signal every 2 seconds.

- Watch physical hardware — LEDs should change based on camera view.

- Check Supabase Table Editor — traffic_events rows should appear every 2 seconds.

- Test override: POST to http://localhost:5000/override with {"signal":"RED","speed_limit":20}. LEDs should immediately go RED. After 60s, AI resumes.

CLAUDE CODE PROMPT — PHASE 5A — Main Orchestration Loop

Create the main orchestration system for the Adaptive Traffic Signal System laptop side.
IMPORTS from previous phases: vision.py, fusion.py, decision.py
FILES TO CREATE:
1. config.py:
   - CAM_IP: ESP32-CAM IP address (from config or .env)
   - DEV_IP: ESP32 Dev Board IP address
   - POLL_INTERVAL: 2 (seconds between each loop)
   - OVERRIDE_DURATION: 60 (seconds override lasts)
   - SUPABASE_URL: from .env
   - SUPABASE_KEY: from .env
   - DEV_CMD_URL: f"http://{DEV_IP}/cmd"
2. supabase_client.py:
   - init_supabase() using supabase-py library
   - log_event(car_count, queue_level, traffic_score, signal, speed_limit, is_override) -> inserts row to traffic_events
   - update_heartbeat() -> upserts key="heartbeat", value=current ISO timestamp to system_state
   - update_tunnel_url(url: str) -> upserts key="tunnel_url" to system_state
   - All functions handle exceptions gracefully (log error, do not crash main loop)
3. api_server.py (Flask):
   - POST /override: accepts {"signal":"RED","speed_limit":20}, sets global override state with expiry
   - POST /clear_override: clears override immediately
   - GET /status: returns current loop state (last_score, last_signal, last_speed, loop_running, override_active)
   - CORS enabled (for Vercel frontend)
   - Runs in daemon thread so it does not block main loop
4. main.py:
   - Loads config
   - Starts Flask api_server in background thread
   - Starts Cloudflare tunnel subprocess, captures URL, saves to Supabase
   - Main loop (runs forever with try/except):
     * Fetch frame from CAM
     * Detect vehicles → car_count
     * Fetch sensors → s25, s50, s75
     * Compute queue_level and traffic_score
     * Compute decision (respecting override)
     * POST command to Dev Board with 3s timeout, retry once on failure
     * Log event to Supabase
     * Update heartbeat
     * Sleep POLL_INTERVAL
   - Graceful shutdown on Ctrl+C: turn all LEDs off (POST signal=RED), show OFFLINE on OLED
   - Print loop summary every iteration: timestamp, car_count, score, signal, speed_limit
5. .env.example:
   - SUPABASE_URL=your_project_url
   - SUPABASE_KEY=your_anon_key
   - CAM_IP=192.168.x.x
   - DEV_IP=192.168.x.x
Requirements: supabase-py, flask, flask-cors, requests, python-dotenv
Use threading for Flask, subprocess for cloudflared.

## PHASE 6 — Local Streamlit Dashboard

### What to expect

After this phase: A browser tab at http://localhost:8501 showing live camera feed, IR sensor status, traffic score, current signal, and speed limit — all auto-refreshing.

CLAUDE CODE PROMPT — PHASE 6A — Streamlit Dashboard

Create a local Streamlit dashboard for the Adaptive Traffic Signal System.
FILE: dashboard.py
Requirements:
1. Auto-refreshes every 2 seconds using streamlit-autorefresh
2. Layout: two columns — left for camera feed, right for metrics
3. LEFT COLUMN:
   - Heading: "Live Camera Feed"
   - Fetches JPEG from ESP32-CAM /stream endpoint (from config.py)
   - Displays as st.image()
   - Shows "Camera offline" placeholder if fetch fails
4. RIGHT COLUMN:
   - Current signal: large coloured badge (red/yellow/green rectangle with text)
   - Current speed limit: large number with "km/h"
   - Traffic score: st.metric() with delta from previous reading
   - Car count: st.metric()
   - IR sensor status: 3 coloured circles (green=clear, red=vehicle) labeled "Near 25%", "Mid 50%", "Far 75%"
   - Last updated timestamp
5. Bottom section: reads last 20 rows from Supabase traffic_events table and shows as st.dataframe()
6. Sidebar: shows CAM IP, Dev Board IP, system status (online/offline based on heartbeat from Supabase)
State: reads current values from Supabase system_state or directly from config.
Use: streamlit, streamlit-autorefresh, supabase-py, requests, PIL
Install command to print at startup: pip install streamlit streamlit-autorefresh supabase requests Pillow

## PHASE 7 — Vercel Next.js Police Dashboard

### What to expect

After this phase: A deployed URL (e.g. https://traffic-ai.vercel.app) that any traffic police officer can open on their phone or laptop to see the live feed, current traffic state, history, and send override commands.

### Prerequisites for Phase 7

- Phases 1-6 fully working and tested.

- Supabase project set up with traffic_events and system_state tables.

- Node.js 18+ installed on laptop.

- Vercel account (free tier is fine).

- Vercel CLI: npm install -g vercel

### Testing Phase 7

- Run locally first: npm run dev → open http://localhost:3000

- Check live feed appears (requires Cloudflare tunnel running and URL in Supabase).

- Check Realtime: keep Supabase table open in another tab — new rows should appear within 2s of main.py loop.

- Test override: click RED button → confirm physical LED changes within 3-5 seconds.

- Deploy: vercel --prod → get your public URL.

- Share URL with demo — anyone can access it from their phone.

CLAUDE CODE PROMPT — PHASE 7A — Next.js Project Setup

Create the complete Next.js project structure for the Traffic Police Dashboard.
SETUP:
- Framework: Next.js 14 with App Router
- Styling: Tailwind CSS
- Database: Supabase (supabase-js v2)
- Charts: Recharts
- Icons: lucide-react
FILES TO CREATE:
1. package.json with all dependencies
2. .env.local.example:
   NEXT_PUBLIC_SUPABASE_URL=
   NEXT_PUBLIC_SUPABASE_ANON_KEY=
   NEXT_PUBLIC_FLASK_API_URL=http://[laptop-ip]:5000
3. lib/supabase.ts: Supabase client initialisation
4. lib/types.ts: TypeScript types for TrafficEvent, SystemState, OverrideCommand
5. app/layout.tsx: root layout with dark theme (bg-gray-950 text-white), title "Traffic AI — Police Dashboard"
6. app/page.tsx: imports and arranges all components
COMPONENTS to scaffold (just the file structure and props, not full implementation):
- components/LiveFeed.tsx — props: tunnelUrl string
- components/TrafficState.tsx — props: event TrafficEvent
- components/OverridePanel.tsx — props: flaskApiUrl string
- components/HistoryChart.tsx — props: events TrafficEvent[]
- components/HistoryTable.tsx — props: events TrafficEvent[]
- components/AlertBanner.tsx — props: score number
- components/SystemHealth.tsx — props: lastHeartbeat string
Create full implementation for layout.tsx and page.tsx.
page.tsx must:
- Fetch tunnel URL from Supabase system_state on load
- Subscribe to Supabase Realtime on traffic_events table
- Maintain state: events array (last 50), currentEvent, isOnline
- Pass appropriate props to each component
- Show loading skeleton on initial load

CLAUDE CODE PROMPT — PHASE 7B — Core Dashboard Components

Implement the core components for the Traffic Police Dashboard Next.js app.
Implement these components fully:
1. components/LiveFeed.tsx:
   - Props: tunnelUrl: string | null
   - Shows live MJPEG stream as <img src={tunnelUrl + "/stream"}>
   - If tunnelUrl is null: show "Camera feed loading..." skeleton
   - If image fails to load: show "Camera offline" with camera-off icon
   - Overlay on top-right: small badge showing "LIVE" with pulsing red dot
   - Rounded corners, full width, max height 400px, object-fit cover
2. components/TrafficState.tsx:
   - Props: event: TrafficEvent | null
   - Large signal indicator: circle with RED/YELLOW/GREEN color, pulsing animation when active
   - Speed limit display: large number (e.g. "40") with "km/h" below, inside a road-sign style circle (white circle, red border)
   - Row of 3 metrics: Car Count, Queue Level, Traffic Score — each as a card with icon
   - All gray skeleton cards if event is null
3. components/OverridePanel.tsx:
   - Props: flaskApiUrl: string
   - Three large signal buttons: RED, YELLOW, GREEN — colored backgrounds
   - Speed limit dropdown: select with options 20, 30, 40, 60, 80
   - "Send Override" button: posts to flaskApiUrl/override
   - "Clear Override" button: posts to flaskApiUrl/clear_override
   - Shows override active status with countdown timer if override is running
   - Disable buttons while request is in flight
4. components/AlertBanner.tsx:
   - Props: score: number
   - If score >= 17: show full-width red flashing banner "HIGH CONGESTION DETECTED — Score: {score}"
   - If score >= 12: show yellow warning banner "Heavy Traffic — Score: {score}"
   - If score < 12: render nothing (null)
   - Use CSS animation for the flashing effect on high congestion
5. components/SystemHealth.tsx:
   - Props: lastHeartbeat: string | null (ISO timestamp)
   - Computes seconds since last heartbeat
   - < 15s: green dot + "System Online" + last updated time
   - 15-60s: yellow dot + "Connection Slow"
   - > 60s or null: red dot + "System Offline"
   - Updates every second using setInterval
Use Tailwind CSS for all styling. Dark theme throughout (gray-900 cards on gray-950 background).

CLAUDE CODE PROMPT — PHASE 7C — Charts, History, and Deployment

Complete the remaining components and deployment configuration for the Traffic Police Dashboard.
1. components/HistoryChart.tsx:
   - Props: events: TrafficEvent[] (last 30 minutes of data)
   - Line chart using Recharts LineChart
   - X axis: time (formatted as HH:MM:SS)
   - Y axis: traffic_score (0 to 25)
   - Line: traffic_score — color changes by value (green <7, yellow 7-12, red >12)
   - Add reference lines at y=7 (dashed yellow, "Moderate") and y=12 (dashed red, "Heavy")
   - Tooltip shows: time, score, signal, car count, speed limit
   - Chart title: "Traffic Score — Last 30 Minutes"
   - Dark theme: gray-800 background, white text, gray-600 grid lines
2. components/HistoryTable.tsx:
   - Props: events: TrafficEvent[]
   - Scrollable table (max height 300px)
   - Columns: Time, Signal (colored badge), Speed Limit, Car Count, Queue, Score, Override (yes/no badge)
   - Most recent row highlighted
   - Alternating row backgrounds (gray-800 / gray-900)
3. next.config.js:
   - Allow images from all domains (for MJPEG stream from Cloudflare tunnel)
   - Set up rewrites if needed for API calls
4. vercel.json:
   - Standard Next.js deployment config
   - Set build command and output directory
5. README.md:
   - Setup instructions
   - Environment variables list
   - How to run locally (npm run dev)
   - How to deploy (vercel --prod)
   - How to connect to running backend (set FLASK_API_URL)
6. Deployment checklist as comments in page.tsx:
   - Set NEXT_PUBLIC_SUPABASE_URL and KEY in Vercel dashboard
   - Set NEXT_PUBLIC_FLASK_API_URL to laptop IP (must be on same network or use ngrok)
   - Enable Realtime on Supabase traffic_events table
   - Confirm Cloudflare tunnel running before demo

# SECTION 6 — MASTER TEST FILE FOR CLAUDE CODE

The following is the complete project specification. Paste this as the FIRST message when starting a new Claude Code session. It gives Claude Code the full context to work on any part of the project.

CLAUDE CODE PROMPT — MASTER PROJECT CONTEXT (paste this first in every Claude Code session)

# ADAPTIVE TRAFFIC SIGNAL SYSTEM — COMPLETE PROJECT SPEC
## Project Summary
A distributed edge-AI traffic management system with:
- ESP32-CAM: camera + IR sensors (flash-once, sealed)
- ESP32 Dev Board: LED traffic lights + OLED speed display
- Laptop: YOLOv8 vision + fusion + decision + Flask API + Supabase publisher
- Vercel: Next.js police dashboard with live feed + override + history
## Hardware Connections
### ESP32-CAM (sealed after Phase 1)
- IR Sensor 1 OUT → GPIO 2 (25% queue / near lane)
- IR Sensor 2 OUT → GPIO 4 (50% queue / mid lane)
- IR Sensor 3 OUT → GPIO 16 (75% queue / far lane)
- All IR VCC → 3.3V, All IR GND → GND
- Power: 5V/2A dedicated supply
- ONLY GPIOs available: 2, 4, 16 (all camera/SD pins locked)
### ESP32 Dev Board (reprogrammable via USB)
- RED LED → 220 Ohm → GPIO 25 → GND
- YELLOW LED → 220 Ohm → GPIO 26 → GND
- GREEN LED → 220 Ohm → GPIO 27 → GND
- SSD1306 OLED: SDA → GPIO 21, SCL → GPIO 22, VCC → 3.3V, GND → GND
- OLED I2C address: 0x3C
## Command JSON (core data contract)
{"signal": "GREEN", "duration": 20, "speed_limit": 40}
- signal: RED / YELLOW / GREEN
- duration: seconds to hold signal
- speed_limit: 20 / 30 / 40 / 60 / 80 (km/h)
## Traffic Score Formula
queue_level = (s25 * 1) + (s50 * 2) + (s75 * 3)   [max=6]
traffic_score = (car_count * 0.6) + (queue_level * 0.4)
## Decision Table
score < 3    → GREEN, 30s, 80 km/h
score 3-7    → GREEN, 25s, 60 km/h
score 7-12   → YELLOW, 20s, 40 km/h
score 12-17  → RED, 20s, 30 km/h
score >= 17  → RED, 30s, 20 km/h
## HTTP Endpoints
### ESP32-CAM (port 80)
GET /stream  → returns JPEG image/jpeg
GET /sensors → returns {"s25":0,"s50":0,"s75":0}
### ESP32 Dev Board (port 80)
POST /cmd    → accepts command JSON, drives LEDs + OLED
             → returns {"status":"ok"} or {"error":"..."}
### Laptop Flask API (port 5000)
POST /override       → {"signal":"RED","speed_limit":20} — overrides AI for 60s
POST /clear_override → clears override immediately
GET  /status         → returns current system state
## Supabase Tables
### traffic_events
id (uuid PK), created_at (timestamptz), car_count (int),
queue_level (int), traffic_score (float), signal (text),
speed_limit (int), is_override (bool)
Realtime: ENABLED
### system_state
key (text PK), value (text), updated_at (timestamptz)
Rows: heartbeat (ISO timestamp), tunnel_url (Cloudflare URL)
## Project File Structure
/esp32cam/
  main.ino, camera_config.h, ir_sensors.h, config.h
/esp32dev/
  main.ino, led_control.h, oled_display.h, config.h
/laptop/
  main.py, vision.py, fusion.py, decision.py,
  api_server.py, supabase_client.py, config.py,
  dashboard.py, requirements.txt, .env.example
  tests/ test_esp32cam.py, test_devboard.py,
         test_vision.py, test_fusion.py
/web/
  (Next.js 14 App Router project)
  app/ layout.tsx, page.tsx
  components/ LiveFeed.tsx, TrafficState.tsx,
              OverridePanel.tsx, HistoryChart.tsx,
              HistoryTable.tsx, AlertBanner.tsx,
              SystemHealth.tsx
  lib/ supabase.ts, types.ts
  package.json, next.config.js, vercel.json
## Phase Order
P1: ESP32-CAM firmware (flash once)
P2: ESP32 Dev Board firmware (reprogrammable)
P3: Laptop vision module (YOLOv8)
P4: Laptop fusion + decision engine
P5: Laptop main loop + Flask + Supabase + Cloudflare
P6: Local Streamlit dashboard
P7: Vercel Next.js police dashboard (last)
## Key Libraries
Arduino: WebServer.h, esp_camera.h, ArduinoJson.h, Adafruit_SSD1306.h, Adafruit_GFX.h, Wire.h
Python: ultralytics, flask, flask-cors, supabase-py, requests, Pillow, python-dotenv, streamlit
Node: next, @supabase/supabase-js, recharts, lucide-react, tailwindcss
## Demo Talking Point
"The ESP32-CAM acts as a sealed edge sensor — it never needs reprogramming.
The laptop is the AI brain. The Dev Board is the physical actuator.
A traffic officer anywhere can open the Vercel URL on their phone,
see the live road feed, current signal, speed limit, and override if needed."

# SECTION 7 — DEBUGGING REFERENCE

## 7.1 — Common Issues and Fixes

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| ESP32-CAM not detected on COM port | GPIO0 not bridged to GND | Bridge GPIO0→GND, press RESET, try again |
| Camera init failed (Serial Monitor) | Wrong board selected or power issue | Select AI-Thinker board, use 5V/2A supply |
| WiFi not connecting | Wrong credentials or 5GHz network | Check SSID/password in config.h, use 2.4GHz |
| OLED blank after upload | Wrong I2C address | Try 0x3C then 0x3D in oled_display.h |
| OLED shows nothing after /cmd | ArduinoJson parse failure | Check Content-Type header is application/json |
| YOLOv8 not detecting cars | Confidence threshold too high or angle | Lower threshold to 0.3, check camera angle |
| Supabase rows not appearing | Wrong API key or RLS enabled | Use anon key, disable RLS on traffic_events |
| Vercel live feed not showing | Cloudflare tunnel not running | Restart cloudflared, update URL in Supabase |
| Override not working from Vercel | CORS error or Flask not reachable | Enable CORS in Flask, check FLASK_API_URL env var |
| traffic_score always 0 | Fusion formula bug or mock data | Run test_fusion.py, check fetch_sensor_state() |

## 7.2 — Expected Serial Monitor Output (ESP32-CAM)

Connecting to WiFi.....

WiFi connected! IP: 192.168.1.105

Camera initialised OK

HTTP server started on port 80

[/stream] Frame captured: 12345 bytes

[/sensors] s25=0 s50=0 s75=1

## 7.3 — Expected Terminal Output (main.py)

[14:22:01] car_count=3 | score=4.2 | signal=GREEN | speed=60km/h

[14:22:03] car_count=5 | score=7.0 | signal=YELLOW | speed=40km/h

[14:22:05] OVERRIDE ACTIVE — signal=RED | speed=20km/h | expires in 58s

[14:22:07] car_count=8 | score=9.4 | signal=YELLOW | speed=40km/h

# SECTION 8 — HACKATHON PITCH SUMMARY

"Our system is a distributed edge-AI traffic management platform. An ESP32-CAM acts as a sealed edge sensor — never reprogrammed — serving live video and IR sensor data over WiFi. A laptop running YOLOv8 computer vision fuses camera data with physical sensor data to compute a real-time traffic score. This drives both an AI-computed signal phase and a dynamic speed limit displayed on a roadside OLED screen. Every decision is logged to Supabase in real time. Traffic officers anywhere in the city can open our Vercel-hosted dashboard on their phone, see the live road feed, monitor current traffic conditions, and override the signal instantly if needed. The system costs under Rs 2000 in hardware and can be deployed at any junction in under 30 minutes."